# Distillation Design Report Project

Benzene-Toluene Binary Separation

Prepared by Zeke van Dresser | July 2026

Designed a conceptual 100 kmol/h benzene–toluene distillation column using material balances and Fenske–Underwood–Gilliland methods; developed a formula-driven Excel model and sized a 20-tray, 1.50 m-diameter column with approximately 1.85 MW condenser and reboiler duties.

| Design Basis | Selected Result |
| --- | --- |
| Feed | 100 kmol/h saturated liquid |
| Separation | 45->95 mol% benzene |
| Method | Fenske-Underwood-Gililland |
| Column | 1.5m ID x ≈13m |
| Internals | 20 sieve trays |
| Heat duties | ≈1.85 MW each |

| Feed flow | 100 | kmol/h |
| --- | --- | --- |
| Feed benzene | 45% | mol% |
| Distillate target | 95% | mol% Benzene |
| Bottom target | 5% | mol% Benzene |
| Relative volatility | 2.4 | -- |

DESIGN STATUS — Conceptual/preliminary. Not suitable for fabrication or operation without rigorous simulation, vendor rating, mechanical design, relief analysis, and professional review.

# Executive Summary

This project develops a preliminary continuous distillation column for a 100 kmol/h saturated-liquid benzene–toluene feed containing 45 mol% benzene.

| Item | Final Preliminary Design |
| --- | --- |
| Products | D = 44.44; B = 55.56 kmol/h |
| Reflux ratio | 3.83 mol/mol |
| Stages | 12.05 theoretical; 20 actual trays |
| Feed location | Tray 11 from the top, preliminary |
| Diameter | 1.5m |
| Height | ≈ 13m |
| Duties | Qc ≈ 1.85 MW; Qr ≈ 1.85 MW |

# Design Basis Assumptions

| Variable | Symbol | Value |
| --- | --- | --- |
| Feed flow | F | 100 kmol/h |
| Feed benzene | zF | 0.45 |
| Distillate benzene | xD | 0.95 |
| Bottoms benzene | xB | 0.05 |
| Feed conditions | Q | 1, saturated liquid |
| Top pressure | Ptop | 101.3 kPa |
| Relative velocity | aBz/Tol | 2.4 |
| Overall tray efficiency | Ea | 0.65 |
| Tray spacing | -- | 0.45m |

1.1 Assumptions

Binary mixture; benzene is the light key and toluene the heavy key.

Constant relative volatility and constant molar overflow.

Total condenser and partial reboiler; reboiler is an equilibrium stage.

Sieve trays; heat losses, heats of mixing, solids, reaction, and noncondensables neglected.

Top-section properties govern preliminary vapor-capacity screening.

# Material Balance

2.1 Overall and component balances

F = D + B

FzF = DxD + BxB

D = F(zF − xB)/(xD − xB) = 100(0.45 − 0.05)/(0.95 − 0.05) = 44.44 kmol/h

B = F − D = 55.56 kmol/h

| Stream | Total | Benzene | Toluene |
| --- | --- | --- | --- |
| Feed | 100 | 45 | 55 |
| Distillate | 44.444 | 42.222 | 2.222 |
| Bottoms | 55.556 | 2.778 | 52.778 |
| Closure | 100 | 45 | 55 |

2.2 Approximate mass rates

| Stream | Average MW | Mass Rate |
| --- | --- | --- |
| Feed | 85.83 kg/kmol | 8,582.7 kg/h |
| Distillate | 78.81 kg/kmol | 3,502.7 kg/h |
| Bottoms | 91.44 kg/kmol | 5,080.0 kg/h |

# Equilibrium-Stage Design

3.1 Fenske minimum stages

Nmin = ln[(xD,LK/xD,HK)(xB,HK/xB,LK)] / ln(αLK/HK)

Nmin = ln[(0.95/0.05)(0.95/0.05)] / ln(2.40) = 6.73 stages

3.2 Underwood minimum reflux

0.45(2.40)/(2.40 − θ) + 0.55(1.00)/(1.00 − θ) = 1

θ = 1.770

Rmin + 1 = 0.95(2.40)/(2.40 − 1.770) + 0.05/(1.00 − 1.770)

Rmin = 2.554; Rselected = 1.50Rmin = 3.831

3.3 Gilliland stage count

X = (R − Rmin)/(R + 1) = 0.2643

Y = 1 − exp{[(1 + 54.4X)/(11 + 117.2X)]·[(X − 1)/√X]} = 0.4080

Y = (N − Nmin)/(N + 1) → N = 12.05 theoretical stages

Nactual = 12.05/0.65 = 18.54 → select 20 actual trays

# Internal Flows and Energy Duties

4.1 Internal molar flows

L = RD = 3.831(44.444) = 170.27 kmol/h

V = (R + 1)D = 214.72 kmol/h

L′ = L + qF = 270.27 kmol/h; V′ = V − (1 − q)F = 214.72 kmol/h

4.2 Heat duties

Using an average overhead latent heat of 31,000 kJ/kmol:

Qc = Vλ = 214.72(31,000)/3600 = 1,849 kW ≈ 1.85 MW

Qr ≈ Qc = 1.85 MW (screening value; sensible heat and losses neglected)

| Utility | Basis | Rate |
| --- | --- | --- |
| Cooling Water | Cp = 4.18 kJ/kg*K; ΔT=10 K | 44.2 kg/s |
| Saturated stream | λ=2,130 kJ/kg | 0.868 kg/s |
| Annual thermal energy | 8,000 h/year | 14,792 MWh(th)/year each |

The reflux ratio controls a major capital–energy tradeoff: lower reflux reduces utility use but requires more stages; higher reflux does the reverse.

The reflux ratio controls a major capital–energy tradeoff: lower reflux reduces utility use but requires more stages; higher reflux does the reverse.

# Column Diameter and Hydraulics

5.1 Vapor loading

ρV = PMW/(RT) = 101.3(79)/[8.314(354)] = 2.72 kg/m³

ṁV = 214.72(79)/3600 = 4.71 kg/s; QV = 4.71/2.72 = 1.73 m³/s

5.2 Area and diameter

Use an allowable active-area vapor velocity of 1.50 m/s and 80% active area.

Aactive = 1.73/1.50 = 1.153 m²; Atotal = 1.153/0.80 = 1.441 m²

Dcalc = √(4Atotal/π) = 1.355 m → select 1.50 m ID

| Item | Value | Assessment |
| --- | --- | --- |
| Selected area | 1.767 m2 | πD²/4 |
| Active area | 1.414 m2 | 80% assumed |
| Active vapor velocity | 1.22 m/s | 81% of screening limit |
| Tray spacing | 0.45m | Preliminary |
| Design margin | ≈19% velocity | Before vendor rating |

Final tray rating must check flooding, entrainment, weeping, downcomer backup, residence time, pressure drop, foaming, and turndown using verified physical properties and vendor correlations

# 6. Height, Pressure Drop, and Specification

Tray-stack height = 20(0.45) = 9.0 m

| Height Element | Allowance |
| --- | --- |
| Tray stack | 9m |
| Top disengagement | 1.5m |
| Bottom surge / disengagement | 1.5m |
| Feed and internal allowances | 1m |
| Tangent-to-tangent total | ≈13.0 m |

6.1 Pressure-drop screening

ΔPtrays = 20 trays(0.50 kPa/tray) = 10.0 kPa

Adding approximately 2 kPa for nozzles and lines gives a preliminary bottom pressure near 113 kPa absolute when the top is at 101 kPa.

6.2 Equipment summary

| Parameter | Preliminary Specification |
| --- | --- |
| Service | Benzene-toluene fractionation |
| Column shell | Vertical; materials and code review required |
| Dimensions | 1.50 m ID × ≈13 m T–T |
| Internals | 20 sieve trays; 0.45m spacing |
| Feed tray | Tray 11 from top |
| Top / bottom pressure | 101 / ≈113 kPa(a) |
| Total condenser | ≈1.85 MW |
| Reboiler | Kettle or thermosiphon, ≈1.85 MW |
| Reflux drum | 5-10 min holdup in detailed design |

# 7. Control, Safety, and Operability

7.1 Basic control structure

| Objective | Manipulated Variable |
| --- | --- |
| Column pressure | Condenser / vent handling |
| Reflux-drum level | Distillate flow |
| Bottoms level | Bottoms flow |
| Top composition / temperature | Reflux flow or ratio |
| Bottom composition / temperature | Reboiler steam flow |
| Feed rate | Feed control valve |

7.2 Hazard review

| Hazard | Preliminary Action |
| --- | --- |
| Flammable release | Closed system, detection, ventilation, ignition control |
| Benzene exposure | Closed sampling, LDAR, PPE, hygiene program |
| Overpressure | Code-sized relief to approved disposal system |
| Loss of cooling | High-pressure alarm/trip; reduce heat input |
| Reboiler failure | Isolation, pressure review, relief protection |
| Static ignition | Bonding, grounding, classified equipment |
| Vacuum on cooldown | Vacuum design or protection |

A final process hazard analysis should cover startup, shutdown, blocked flow, fire exposure, utility failure, control-valve failure, and human factors. Detailed operating procedures belong to the actual facility design.

# 8. Sensitivity, Limitations, and Next Steps

8.1 Reflux sensitivity

| R / Rmin | R | Theoretical Stages* | Relative Duty |
| --- | --- | --- | --- |
| 1.2 | 3.065 | ≈15.8 | 0.84 |
| 1.5 | 3.831 | 12.1 | 1 |
| 2 | 5.108 | ≈9.8 | 1.26 |

*Gilliland estimate. **Relative to selected vapor flow (R+1)D. This illustrates the tradeoff and is not an economic optimum.

8.2 Limitations

- Constant α neglects composition and temperature effects.

- No rigorous enthalpy, pressure, or VLE profile was computed.

- Tray efficiency and capacity are assumed, not vendor-rated.

- Exchanger areas, fouling, approach temperatures, and detailed utility conditions are not designed.

- Mechanical thickness, loads, nozzles, metallurgy, corrosion, relief sizing, emissions, and cost are outside this preliminary scope.

8.3 Recommended next steps

- Build a rigorous steady-state process simulation with a validated property method.

- Optimize feed tray, pressure, and reflux using annualized cost.

- Obtain vendor hydraulic rating at 50–110% throughput.

- Size heat exchangers, drums, pumps, piping, and valves.

- Develop PFD/P&ID and complete relief and hazard analyses.

- Complete mechanical design, emissions review, and cost estimate.

# 9. Conclusions

The preliminary design separates 100 kmol/h of a 45 mol% benzene feed into 95 mol% benzene distillate and 5 mol% benzene bottoms. The products are 44.44 kmol/h distillate and 55.56 kmol/h bottoms, with 93.83% benzene recovery overhead.

Shortcut calculations give Rmin = 2.55 and 12.05 theoretical stages at R = 3.83. Applying 65% efficiency

10. Calculation Summary

| Calculations | Result |
| --- | --- |
| D / B | 44.44 / 55.56 kmol/h |
| Nmin / Rmin | 6.73 / 2.554 |
| Selected R | 3.831 |
| Theoretical stages | 12.05 |
| Actual trays selected | 20 |
| Internal vapor | 214.72 kmol/h |
| Diameter | 1.5m |
| Height | ≈13.0m |
| Condenser | ≈1.85 MW |
| Reboiler | 1.85 MW |

Methods and References

- Fenske equation: minimum stages at total reflux.

- Underwood equations: minimum reflux.

- Gilliland correlation with Eduljee explicit approximation: stage count.

- Ideal-gas density and superficial-velocity screening: preliminary diameter.

- Property values are representative estimates and must be verified in detailed design.